# Weather Forecast Pipeline

## Kiến trúc hệ thống

Raspberry Pi 5          MQTT Broker          Cloud Server              Unity

┌─────────────┐      ┌────────────┐      ┌──────────────────┐      ┌────────────┐

│ Sensors     │      │ Mosquitto  │      │ mqtt_listener    │      │ Digital    │

│ → CSV 48h   │─MQTT─│ topic:     │─────▶│ → last_48h.csv   │      │ Twin 3D    │

│ → publish   │      │ weather/   │      │ → inference.py   │◀─GET─│ poll /api/ │

│             │      │ data       │      │ → forecast.json  │      │ forecast   │

└─────────────┘      └────────────┘      │ → Flask API :5000│      └────────────┘

└──────────────────┘

## Cấu trúc thư mục

pipeline/

├── edge/

│   └── edge_publisher.py     # Chạy trên Raspberry Pi

├── server/

│   ├── server.py             # MQTT subscriber + Flask API

│   └── inference.py          # Copy file inference.py của bạn vào đây

├── unity/

│   └── WeatherForecastClient.cs  # Script C# cho Unity

├── models/                   # Đặt model files vào đây

│   ├── tcn_model.pth

│   ├── arima_model.pkl

│   └── ensemble_metrics.json (optional)

├── data/                     # Auto-generated

│   ├── last_48h.csv

│   ├── forecast_result.json

│   └── history/

├── mosquitto/config/

│   └── mosquitto.conf

├── docker-compose.yml

├── Dockerfile

├── requirements.txt

└── test_pipeline.py

## Quick Start

### Bước 1: Setup Server

# Clone/copy project

cd pipeline

# Copy inference.py và models vào đúng chỗ

cp /path/to/inference.py server/

cp /path/to/tcn_model.pth models/

cp /path/to/arima_model.pkl models/

# Cài dependencies

pip install -r requirements.txt

### Bước 2: Chạy bằng Docker (recommended)

# Chạy MQTT broker + Server

docker-compose up -d

# Xem logs

docker-compose logs -f forecast_server

### Bước 2 (alt): Chạy thủ công

# Terminal 1: MQTT broker

docker run -d -p 1883:1883 -v ./mosquitto/config:/mosquitto/config eclipse-mosquitto:2

# Terminal 2: Server

python server/server.py \

--broker localhost \

--tcn models/tcn_model.pth \

--arima models/arima_model.pkl \

--api-port 5000

### Bước 3: Test pipeline

# Tạo CSV giả lập + upload + kiểm tra

python test_pipeline.py --server http://localhost:5000

### Bước 4: Chạy Edge (Raspberry Pi)

# Trên Raspberry Pi:

pip install paho-mqtt numpy

# Mode mô phỏng (test)

python edge/edge_publisher.py \

--broker <server-ip> \

--simulate \

--init-buffer \

--interval 60

# Mode cảm biến thật

python edge/edge_publisher.py \

--broker <server-ip> \

--interval 3600

### Bước 5: Unity

- Copy unity/WeatherForecastClient.cs vào Unity project

- Tạo Empty GameObject → gắn script

- Set serverUrl = "http://<server-ip>:5000"

- Trong script khác, subscribe event:

void OnEnable()

{

WeatherForecastClient.Instance.OnForecastUpdated += OnNewForecast;

}

void OnNewForecast(ForecastResult forecast)

{

var next = forecast.forecast[0];

Debug.Log($"Temp h+1: {next.temperature}°C, Rain: {next.precipitation}mm");

// Cập nhật Digital Twin scene ở đây

UpdateWeatherVisualization(next);

}

## API Endpoints

| Method | Endpoint | Mô tả |
| --- | --- | --- |
| GET | /api/forecast | Toàn bộ forecast 6h |
| GET | /api/forecast/latest | Chỉ bước h+1 |
| GET | /api/forecast/step/3 | Bước cụ thể (1-6) |
| GET | /api/current | Dữ liệu sensor mới nhất |
| GET | /api/status | Health check |
| POST | /api/trigger | Trigger inference thủ công |
| POST | /api/upload_csv | Upload CSV (thay MQTT khi test) |

## Ví dụ response /api/forecast

{

"generated_at": "2026-04-04T15:00:00",

"based_on_data_until": "2026-04-04T14:00:00",

"horizon_hours": 6,

"model_used": "TCN+ARIMA ensemble",

"forecast": [

{

"timestamp": "2026-04-04T15:00:00",

"horizon_step": 1,

"temperature": 31.25,

"humidity": 72.8,

"wind_speed": 12.5,

"precipitation": 0.02,

"pressure": 1008.5,

"...": "..."

}

]

}

## Luồng dữ liệu chi tiết

- Raspberry Pi đọc sensor mỗi giờ → append vào buffer 48 hàng

- Buffer đầy → serialize thành CSV string → publish MQTT topic weather/data

- Server subscriber nhận message → parse JSON → lưu last_48h.csv

- Server gọi inference.py --csv last_48h.csv → output forecast_result.json

- Flask API serve JSON tại /api/forecast

- Unity poll GET mỗi 30s → parse JSON → cập nhật Digital Twin scene

python3 server/server.py     --broker localhost     --mqtt-port 1883     --tcn models/tcn_model.pth     --arima models/arima_model.pkl     --inference-script server/inference.py     --ensemble_json models/ensemble_stacking_metrics.json     --api-port 5000

python3 server/server.py 
--broker localhost 
--tcn models/tcn_model.pth 
--lstm models/lstm_model.pth 
--arima models/arima_model.pkl 
--ensemble_json_tcn models/ensemble_stacking_metrics.json 
--ensemble_json_lstm models/ensemble_lstm_SGN_metrics.json 
--inference-script-tcn server/inference.py 
--inference-script-lstm server/inference_lstm_arima.py 
--api-port 5000

cd server

python3 server_dual_tcn.py \

--broker   localhost \

--tcn      ../models/tcn_model_24h.pth \

--tcn_hard ../models/tcn_hard_model_24h.pth \

--api-port 5001

# ===== TCN + ARIMA =====

curl.exe http://192.168.25.129:5000/api/tcn/forecast curl.exe http://192.168.25.129:5000/api/tcn/forecast/latest curl.exe http://192.168.25.129:5000/api/tcn/forecast/step/1

# ===== LSTM + ARIMA =====

curl.exe http://192.168.25.129:5000/api/lstm/forecast curl.exe http://192.168.25.129:5000/api/lstm/forecast/latest curl.exe http://192.168.25.129:5000/api/lstm/forecast/step/1

# ===== So sánh 2 model =====

curl.exe http://192.168.25.129:5000/api/compare

# ===== Chung =====

curl.exe http://192.168.25.129:5000/api/current curl.exe http://192.168.25.129:5000/api/status curl.exe http://192.168.25.129:5000/api/forecast          # backward compatible = TCN curl.exe -X POST -F "file=@test.csv" http://192.168.25.129:5000/api/upload_csv curl.exe -X POST http://192.168.25.129:5000/api/trigger